LAMA –

tooL for Air exposure Modelling& Assessment

Software documentation & User’s guide

Version 1.1, 2011-02-07

Lydia Gerharza, Aileen Yangb, Miranda Lohcd, Alexandra Kuhne

a Institute for Geoinformatics, University of Münster, Germany

b Norwegian Institute for Air Research, Kjeller, Norway

c University of Arizona, Tucson, AZ

dNational Institute for Health and Welfare, Kuopio, Finland

e Institute for Energy Economics and the Rational Use of Energy, 
University of Stuttgart, Germany

Table of Contents

1.	Introduction	4

2.	Model overview	4

2.1	Model structure	4

2.2	Uncertainty and Variability	7

3.	Model Parameters	8

3.1	Model Parameters	8

3.2	Other Parameters	9

4.	Model Input	9

4.1	Time use	9

4.2	Population	10

4.3	Concentration	10

4.4	Urban increment	11

5.	Model Output	11

5.1	Separation of uncertainty and variability	11

5.2	Country-based exposure – diaries	12

5.3	Country-based exposure – subgroups	12

5.4	Grid-based exposure – subgroups	12

5.5	Visualisation	13

5.6	Further data analysis	13

6.	Software details	13

6.1	Local files	13

6.1.1	Input Files	13

6.1.2	Output Files	14

6.1.3	LAMA Logfile	15

6.2	Database	15

7.	Quick start tutorial	17

7.1	Prerequisites	17

7.2	Running the model	17

8.	Final remarks & Known Issues	22

9.	References	22

10.	Contact	24

# Introduction

Currently, the estimation of health effects from air pollutants is based on concentration response functions (CRFs). These CRFs link the ambient concentrations of an air pollutant to health effects that are caused through this pollutant. This ignores the fact that the ambient concentration is not the true exposure of individuals as people tend to spent most of their time indoors in. The true exposure is related to the concentration they are exposed to during the time spending in different microenvironments (e.g. cars, indoors, outdoors). By using the ambient concentration and not the exposure of people as metric health effect estimations can be biased and probably lead to an underestimation of the effects. Furthermore only policy measures that focus on reduction of outdoor emissions and thus concentration can be evaluated if individual time activity patterns are ignored.

LAMA is a tool for estimating exposure to air pollutants in a population by integration of outdoor air pollution models, time use patterns and an indoor model. It was developed within HEIMTSA (Health and Environment Integrated Methodology and Toolbox for Scenario Assessment), an integrated project funded by the European Commission under the 6th framework. The estimated exposure consists of the exposure due to outdoor sources plus the exposure due to indoor sources. This total exposure is spatially distributed due to varying outdoor concentrations, population density and behaviour of humans. The results will be in the form of population averages that can be calculated on a grid or country level (by weighting with the appropriate population). The methodology implemented in LAMA can be used for different air pollutants but has currently only been tested for PM2.5 and so far only input data for PM2.5 is available.

# Model overview

## Model structure

This chapter gives a brief overview over the model theory behind the software. For further details of the model structure please refer to the HEIMTSA wiki: http://heande.opasnet.org/wiki/HI:Policy_for_PM2.5_Exposure

In LAMA the exposure is modelled using a microenvironmental approach. The time spent by a person is classified into microenvironments (MEs) for which the concentrations are estimated. The 5 microenvironments used in LAMA are:

- work & school

- home

- travel & commute

- outdoor

- other (unspecified location)

“Other” includes indoor and outdoor locations if it is not clear from the time-activity data where certain activities take place or the environment fits into none of the other categories (e.g. shopping, meals, playing and watching sports, dance, restaurants, church). In this case for each individual of the time-activity diaries, while sampling, 50 % of the time is assigned to “outdoor” and 50 % to “home”. Time activity data is utilised in the format of daily dairies of aggregated time spent in the defined microenvironments.

The concentration per microenvironment is estimated based on the mass-balance indoor model under steady state, instantaneous and homogenous mixing conditions (Koutrakis, 1992 )

| cin=coutpa+i=1nsiva+k | (1) |
| --- | --- |

Where: 	cin = indoor concentration [µg m-3]

cout = outdoor concentration [µg m-3]

p = penetration factor

a= air exchange rate [h-1]

k = decay rate [h-1]

si = emission rate for source i [µg h-1]

v = room volume [m³]

Within this mass-balance model, indoor and outdoor source contributions can be separated:

| cin=coutf+cin¦s | (2) |
| --- | --- |
| f=paa+k | (2a) |
| cin¦s=i=1nsia+kv | (2b) |

Where: 	f = infiltration factor, i.e. percentage of outdoor concentration reaching the indoor environment

cin|s = indoor concentration caused by indoor sources

Factor f is dependent on the microenvironment. For “outdoor” f it is set to 1 as no infiltration takes place and cin|s is 0. For “travel & commuting” f can be substituted by the traffic factor e which is the ratio between the concentration one is exposed to while in transport (based on studies examining on-road exposures via car, bus, bike, and walking) and the ambient background concentration:

| e=ctranscback | (3) |
| --- | --- |

Where 		e = traffic factor

ctrans = concentration a person is exposed to while in transport

cback = urban background fixed site concentration

As air pollution often is higher in highly populated areas, it is important to take into account elevated concentrations for urban areas. If the spatial resolution of the ambient concentration, estimated e.g. by an atmospheric model as in the HEIMTSA project (EMEP grid, 50km x 50km),coarse,   an urban concentration increment curb is taken into account for people living in urban areas.

Based on those equations, LAMA combines the human time activity data with the concentration data and microenvironment-specific parameters to estimate individual exposure. The final calculation of exposure can be performed either on a diary basis (for each country, eq. 4a), as subgroup average for each grid cell (eq. 4b) or as subgroup average for each country (eq. 4c, derived from 4a or 4b):

| Ec,d=1i=1npopii=1npopi1j=1mtd,jj=1mtd,jfjcout,i+curb,i+cin¦s,j | (4a) |
| --- | --- |
| Ei,g=1qgd=1qg1j=1mtd,jj=1mtd,jfjcout,i+curb,i+cin¦s,j | (4b) |
| Ec,g=1qgd=1qgEc,d=1i=1npopii=1npopiEi,g | (4c) |

Where :	E = exposure [µg m-3]

c = country

i = grid cell

g = subgroup

d = individual diary of subgroup g

q = number of diaries per subgroup g

f = infiltration or traffic enrichment factor (see eq. 3)

pop = population per subgroup and grid cell

t = time fraction spent in a microenvironment

j = microenvironment

curb = urban increment for cities in grid cell i [µg m-3], this factor was only added to a number of individuals according to the percentage of people living in an urban area in the grid cell

## Uncertainty and Variability

In LAMA, there are two main parameter types that have inherent uncertainty and variability – the parameters related to estimating indoor concentrations and the time activity diary data. The greatest uncertainty is likely in the indoor parameters. Therefore uncertainty is explored by simulating a set of realisations for each diary, based on the distributions of the indoor parameters. To account for the uncertainty in the indoor parameter estimations (i.e., a, k, p, v, s) each parameter is represented by a distribution instead of single values. In LAMA, Monte Carlo sampling is used to draw a large number of realisations from each of the parameter distributions. The variability between the diaries is separated from the uncertainty of the true parameters for each diary within LAMA by keeping both sample dimensions (r realisations and q diaries) separately. This gives two ways to look at the results: 1) the variability between exposures estimated per diary (averages over the realisations) and 2) the uncertainty for each diary result between realisations (or the uncertainty of the population average between realisations).

Figure 1: Presentation of the separation of uncertainty and variability. D represents a diary, E represents an exposure distribution for each diary.

Uncertainties due to spatial extrapolation of the indoor parameters, limited representativeness of the MTUS data for the whole country and policy assumptions are acknowledged but cannot be quantified at this stage.

# Model Parameters

This section gives a brief overview of the necessary model parameters. More details about the indoor model parameters can be found in the wiki.

## Model Parameters

Parameters described by distributions (microenvironments):

- Air exchange a (home/work)

- Penetration factor p (home/work)

- Decay rate k (home/work)

- Room volume v (home/work)

- Background emission source s_bgr (home)

- ETS emission source s_cig (home)

- Biomass burning emission source s_bio (home)

- # smoked cigarettes per person n_cig (home)

- Traffic enrichment factor e (travel)

Parameters representing fractions of the population:

- Fraction of people exposed to Biomass burning f_bio (home)

- Fraction of people exposed to ETS, including passive and active smokers f_cig (home)

Scenarios based on policies can change these parameters. Therefore each parameter value is connected to a certain scenario.

## Simulation Parameters

The parameters used for the simulation are selected for each model run by the user. This are

- Country (single or whole EU-30)

- Scenario– scenarios affect input parameters, time use and concentration

- Pollutant (currently only PM2.5)

- Number of Monte Carlo iterations

- Spatial result type (grid or country based )

- Population result type (individual diary days, subgroups, total population)

- Uncertainty result storage: mean and standard deviation or Monte Carlo realisations

- Indoor sources: Excluding indoor background sources (yes/no), Excluding ETS sources (yes/no), Excluding Biomass sources (yes/no)

- Visualisation storage (only for grid based results): mean and standard deviation or quantiles maps

- Start Aguila visualisation tool with maps (only if Aguila is installed and configured on the local computer)

# Model Input

## Time use

The data used for time use in LAMA it the Multinational Time Use Survey (MTUS) data (see http://www.timeuse.org/mtus/). Each person has to register at the MTUS website to get access to the data for a specific research purpose only. After registration, you should download the W552 and W553 csv files which contain data for a number for countries. In order to use the MTUS data with LAMA you have to do some preparations:

- Copy the data from the files W552 and W553 into one file named timeuse.csv and copy this into the LAMA/input folder.

- Add a column named “countryID” and add the names of the countries (ISO2 country codes) in the new column: DK(for no. 2), FR(3), NL(4), UK (6), IT(18), DE(22), AT(23), ES(35), NO(5), SI(34)

- Remove older diaries (e.g. older than 1990) and non-European countries. This will reduce the time used to read the file later. Non-European countries are Australia (19) and USA (7).

You do not have to add columns for subgroups and microenvironments for the MTUS data, as the program will do the grouping automatically and increases time during the parser process only marginally.

You can also use other time use data with LAMA, e.g. the free US NHAPS data. Make sure that the time use data contains one diary, i.e. individual person, per row and is compliant with the table structure as shown in tab. 1. The file may contain more columns than this, but must contain columns for country name, age, sex and age as well as the time spend in the 5 microenvironments have to be in the table. You probably have to regroup your time use data to the microenvironments used in LAMA. Make sure that the sum of these 5 columns is 1440 (min/day).  This assumes that each diary is for a 24 hour (1440 min) time period.

Table 1. Format of mtus.csv file.

| countryID | age | sex | emp | home | work | travel | other | outdoor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| String (ISO2 coded) | Integer | Integer (1=male, 2=female) | Integer (0=nonwork, 1=work) | Integer (0-1440) | Integer (0-1440) | Integer (0-1440) | Integer (0-1440) | Integer (0-1440) |

## Population

The population data needs to be defined on the same grid as the concentration data (in this case the EMEP 50 km x 50 km grid).  Population data is stratified by population subgroups to allow exposure to be calculated for each group. Subgroups can in principle be defined as needed by the user. The default by LAMA is females/males aged 0-14, working females/males aged 15-64, non-working females/males aged 15-64, females/males aged 65+. Summing up all subgroups gives the total population of a grid cell. For the structure of the population file see the data base schema (fig. 1).

## Concentration

In the HEIMTSA and INTARESE projects concentration data is provided by chemical transport models from Met.no, ENPC and INERIS, or by the EcoSense model from USTUTT. Concentration data could be taken from other sources as long as it is available on the same grid as the population data (in this case EMEP 50 km x 50 km grid). The concentration file should include all the grid cells over Europe. Within the LAMA model run the respective cells for each country are selected by using the population data.

Concentrations are given as annual average concentration (at least for e.g. PM2.5 – for ozone other means would be relevant, e.g. SOMO35).

Scenarios not only influence the input parameters (e.g. p, a, k, v) for LAMA but also the concentrations (via a change in the emissions). This, however, is not directly calculated in LAMA , but  is estimated as part of an external model and then input to LAMA.

## Urban increment

The urban increment is derived from a quantitative relationship between rural (background) and urban concentrations as measured by air monitoring stations. It represents the additional urban concentration added to the background which was estimated by the chemical transport models (5.2).

The urban background increment for PM10 is estimated using a functional relationship between emissions, city size and average wind speed. For this purpose, observational data for urban and rural background stations for PM10 were retrieved from the European Air Quality database (Airbase 2008). To obtain the relationship between predictor variables and the criterion variable — in this case the measured urban increment — a multiple regression analysis was carried out, using the following formulation:

| Ci urban=ϖi+φiEiUEAUE⋅uavg+γ Ci rural | (4) |
| --- | --- |

With: 		Ci urban = Urban increment of pollutant i.

EiUE = Total emission of pollutant i within an urban entity in tons.

AUE = Urban entity area in km2.

uavg = Urban entity average wind speed in m/s.

Ci rural = Rural background concentration of pollutant i in µg/m3 (only for PM10).

ωi, φi, and γi = Multiple-regression parameters for pollutant i

An urban increment model for PM2.5 could not be estimated directly due to the limited availability of valid measurements for this pollutant. Therefore the PM2.5 increment was derived by using the ratio between PM2.5 and PM10.  Thus, although it is recognized that the ratio of PM2.5 to PM10 depends on factors which vary both spatially and temporally, a value of 0.70 for the PM2.5/PM10 urban background ratio is considered to be a reasonable assumption (Querol et al., 2004).

# Model Output

## Separation of uncertainty and variability

As uncertainty of the model parameters and variability of the individuals in a population are kept separately, the results are always two dimensional. These dimensions are r Monte Carlo samples from the parameter distributions and q diaries, subgroups or subgroups per grid cells, depending on the results type (see next three chapters). The resulting r realisations of the population with q individuals can be visualised as cumulative probability curves for each population (fig. 2, black curves). Variability within the individuals of a population is represented by the slope of the curve. Uncertainty is shown by the difference between the curves. For further calculations (i.e. health effects) we do not distinguish the individuals but make a distribution of the whole sample size (blue curve). This includes the variability and the uncertainty.

Figure 2. Schematic example of variability within and uncertainty between curves.

## Country-based exposure – diaries

Eq. 4a gives for each country the exposure per diary averaged over the grid cell concentrations. That means, for each diary, the exposure in each grid cell is calculated and averaged over the grid cells per country afterwards, weighted by the population living in each grid cell. Thus, higher populated grid cells get higher weights. This result gives an estimate of the variability of exposure within the total or subgroup population of a country as each diary is preserved as individual result. However it has to be noted here that LAMA does not model the exposures of a cohort or population of hypothetical individuals. Individual time activity patterns over a year are not reconstructed. Thus, the variability between single diary days not only includes variability between individuals but also variability between different days of one individual, i.e. intra-individual variability.

## Country-based exposure – subgroups

As the results are not interpretable per diary as one diary does not equal an individual, it is recommended to average the results over subgroups for further analysis. Thus, LAMA provides the summarised exposure per subgroups per country. As shown in eq. 4c, this can be done by averaging over diaries per subgroup or grid cells per country. The aggregated output also leads to a significantly reduced calculation time for LAMA compared to the diaries results. Results are again two dimensional with r realisations and the number of subgroups per country.

## Grid-based exposure – subgroups

Compared to the country based exposure, the grid based exposure (eq. 4b) gives an overview of the spatial distribution of exposures. The result is the exposure per grid cell averaged over individual diaries per population subgroup. Exposures can be compared by looking at different grid cells, or by looking at different subgroups in one grid cell. Averaging over grid cells weighted by the population yields again a country-based exposure per subgroup (eq. 4c).

## Exposure – total population

For country based as well as grid based results the exposure can be summarised over the total population. This is done by averaging the subgroup exposure by weighting each with the percentage of the total population in the grid cell or the country.

## Visualisation

For grid based results maps are stored for visualisation. Map files are stored in the ESRI Ascii Grid format which is supported by the Geospatial Data Abstraction Library (www.gdal.org). The uncertainty represented by the parameter realisations can be stored as grid maps showing mean and standard deviation or different quantiles. It is possible to automatically start Aguila (http://pcraster.sourceforge.net/Aguila/trunk/) with from LAMA showing the respective maps. This allows exploring the uncertainties and variability in the data over whole Europe. Visualisation of different subgroups and for country averages is currently not supported.

## Further data analysis

For further analysis, the data can be either retrieved from the database or by using the written csv files. LAMA currently does not provide any functionality for further analysis like calculating averages or new distributions. It will be necessary to feed the data again into a statistics program or a new model. For visualisation in R a number of scripts are provided which can be customised.

# Software details

## Local files

### Input Files

The input data, except the time use data, comes with the LAMA distribution as comma separated value (csv) files. Each file consists of a header (first line) which includes the column names and the body (line 2+) which comprises the data. The formatting is similar to the tables in the database schema (see figure 1). You can change the values of the input data if you like but you should not change names of the columns, separators (usually comma) or the folder structure within the LAMA folder. The LAMA folder needs to contain the files in the following structure:

- \input\data:

  - concentration.csv

  - parameter_distribution.csv

  - parameter_fraction.csv

  - population.csv

  - timeuse.csv (PLEASE NOTE: this file is not included in the LAMA distribution and has to be downloaded and/or created separately! See Chapter 4.1 for more details.)

  - urban_increment.csv

- \input\metadata:

  - countries.csv

  - microenvironment.csv

  - parameter.csv

  - pollutant.csv

  - scenario.csv

  - subgroup.csv

### Output Files

Output files with the exposure results are written as csv files as well or as asc files for the maps. The results are stored in folders named by the scenario ids and either a folder named “grid” or “country” to indicate the different result types. The names of the output files include the country, the scenario ID, number of Monte Carlo iterations and “grid”, “subgroups” or “diaries” to indicate the type of result according to eq. 4a, 4c and 4b. Additionally, the values of the parameter samples for each iteration are written into a csv file marked with “para”. Here you can check for each realisation which parameters were used to yield the respective results. For example

AT_ 2010BAU_ _PM2.5_i1000_diaries.csv

contains results for individual diaries in Austria, for PM2.5 exposure and 1000 iterations. 8b24cf5b-5046-4db7-a7dd-e1417385e406 is the unique identifier for this model run.

AT_2010BAU_PM2.5_i1000_para.csv

contains the parameter realisations used for each iteration of this run.

To document the simulation details a simulation info file is written into the \output\logs\ folder. The name consists of the country ID plus the simulation ID. It contains information about the simulation parameters and some test outputs from the input data to check if any problems during the parser process occurred. For our example model run the simulation log file would be:

AT_2010BAU_PM2.5_i1000_diaries.txt

The grid, subgroup or diaries output file has the following structure:

- Grid file: Emep_ID, Group_ID, iterations (r times, see Chapter 2.2)

- Subgroup file:Group_ID, iterations (r times, see Chapter 2.2)

- Diaries file: Group_ID, diary_ID, ETS, biomass, iterations (r times, see Chapter 2.2)

- Visualisation file: mean or standard deviation or quantiles as Ascii Grid files

The grid file structure means that per grid cell i, per subgroup g and per uncertainty MC iteration r (sampling from distributions of indoor parameters) an average of the exposure of all individual diaries is given. This is corresponding to the Equation (4b).

The subgroup file structure means that per subgroup g the exposure for each uncertainty MC iteration k (sampling from distributions of indoor parameters), summed over all grid cells of a country c, is given. This corresponds to the Equation (4c).

The diaries file structure means that per individual diary the exposure for each uncertainty MC iteration k (sampling from distributions of indoor parameters), summed over all grid cells of a country c, is given. This corresponds to the Equation (4a).

The Ascii Grid maps can be visualised with common GIS tools with GDAL support. For the EMEP data, files are written in the ETRS LAEA projected coordinate system.

### LAMA Logfile

LAMA maintains a logfile to store information and output from the program. This can be info messages thrown by LAMA or exception reports captured if an error occurs during the program operation. After a successful run, there will only be 1 line in the logfile proving the successful model run. After a crash of the program the logfile can help to identify the cause of the crash as it logs errors thrown by LAMA and should therefore be included in error reporting.

## Database

The database structure including all tables and the dependencies is shown in Figure . You do not need to create any tables as this is done within the program when you choose the option “fill database”. The tables are grouped into four thematic areas: time use (MTUS) data, grid (EMEP) data, Model Parameters and Results. The content of the tables is explained in the next three chapters.

Figure 3. Database schema

# Quick start tutorial

## Prerequisites

LAMA can be downloaded as zip file. Extract the folder in the LAMA.zip archive to a directory of your choice. By default, the LAMA folder is C:\LAMA but could in principle be anywhere. Make sure that you do not change the folder and file structure within the LAMA folder itself as LAMA needs to know where to find the different files.

LAMA is written in Java. It is possible and recommended to use LAMA with a database, as the amount of diary results can be considerably large. To run the program, you need:

- Java runtime environment, JRE on http://www.java.com/en/download/index.jsp

- If you want to use a database, choose the system you prefer, either

  - PostgreSQL (recommended) on http://www.postgresql.org/download/

  - MySQL on http://www.mysql.com/downloads/

Please follow the installation instructions of the respective software package. For the database system, you should create a new and empty database which will be filled by the LAMA model. The SQL command to create a new database (here called LAMAdb) is:

- CREATE DATABASE LAMAdb;

The database service can run locally or on a server. On a server it is important that the TCP/IP connection is enabled over a port number (default is 3306 for MySQL and 5432 for PostgreSQL but you can also choose another port). If the database runs locally, the port number can be omitted for MySQL but not for PostgreSQL. Also ensure that your database service has been started when using LAMA. The JAVA connector libraries for PostgreSQL and MySQL are already included in the distribution.

The program has been developed on a Windows 7 Enterprise Edition 64-bit System with Java 1.6, Update 19 and PostgreSQL 8.4.2. As the model runs are resource intensive, the main memory of the PC used should be at minimum 2 GB.

For visualisation with Aguila, the Aguila software has to be installed correctly. You can find the software details and installers on http://pcraster.sourceforge.net/Aguila/trunk/. It is necessary to set the path in the Windows environment variable for execution of Aguila automatically from LAMA.

## Running the model

LAMA is available as source code written in Java 1.6 and as executable jar file. After installing the prerequisites you can execute the LAMA.jar by using the “startLAMA.bat” file. Alternatively you can start LAMA from the command line:

java –Xmx1024M –jar C:\LAMA\LAMA.jar

If you have extracted the LAMA folder in another directory, change this line accordingly in the bat-file. The start-up of the model is a 3 step configuration GUI to specify your data sources and parameters for the model run.

Data source configuration:

In the first step you have to specify if you want to read your input data

a) from csv files (which come with this distribution, except the timeuse file)

b) from the database if it’s filled already

c) read the input from csv files and fill the database with the parsed data

The last option can take some time (~ 1.5 hours on a PostgreSQL system), as the insert of the time use data into the database is resource consuming. Note that the whole database will be initiated automatically if you choose “fill database”! This means that existing LAMA tables, especially result tables, will be cleared and data previously written by LAMA into this database will be lost! If you want to prevent this, export the data to another database or create a new one to be filled by LAMA. If the filling finishes successfully, you can access the data from the database directly when you start LAMA the next time. For the present model run, the desired data is already loaded from the csv files.

For the output data you can specify whether you want to have it written into csv files in the output directory of your LAMA folder or into the database. The latter option is only possible if the database has already been initiated and filled (as the results correspond to certain input data). So choosing “read from csv” and “write to database” will throw an error, as the program assumes that the database does not exist.

Csv source configuration:

If you chose read/write from/to database or if you extracted the LAMA.zip archive to the default LAMA folder “C:\LAMA”, this option will not be shown to you. Otherwise, you have to select the local directory where you unpacked your LAMA folder. By default, this is C:\LAMA but could in principle be anywhere. Make sure that you do not change the folder and file structure within the LAMA folder as LAMA needs to know where to find the different tables. The folder will be checked if all 12 csv input files are in the /input and /input/simple folders. If LAMA cannot find all files an error message will occur and you should check if all files are available as listed in Chapter 6.1.

Database source configuration:

If you chose read/write from/to csv, this option will not be shown to you. When using a database you have to specify in this step the database driver, your database URL (with port number) and database name (in this example it is running locally on the computer). Furthermore, LAMA needs a valid database user with editing rights (usually the owner of the database) and the password. For the URL you have to enter the port over which the database is accessed and the name of the database (here “LAMAdb”). The username and password are NOT stored in LAMA for safety reasons and thus have to be entered again each time LAMA is launched. LAMA will check the database connection before the next step is invoked.

Simulation configuration:

Simulation tab

Finally, the details of the simulation have to be specified. Each simulation can be run for one country, one pollutant (currently only PM2.5) and one scenario only. Selecting ‘all’ from the country drop-down box allows to execute LAMA for all European countries with the selected parameters in succession. Similarly, selecting ‘all’ from the scenarios list will yield subsequent execution of all scenarios. However results will still be stored in separate country and scenario files. The number of iterations should not be too high (max. 1,000 for diaries and 100,000 for grid and subgroup results), as LAMA can only process a limited number of values at one time. A reasonable number to start with is 500 and 10,000 iterations, respectively.

Results tab

For the results you can specify the spatial and population aggregation level and the representation of uncertainties you want to store your results as (see chapter 5). Spatial level can be grid or country based, population level is diaries, subgroups or total population and uncertainties can be stored as Monte Carlo realisations or as mean and standard deviation of these realisations.

Sources tab

As last option for the model run, the calculation can be performed with or without indoor sources or only without ETS sources. Excluding indoor sources completely would yield only the outdoor air contribution to the exposure (see eq. 2/2a). Excluding ETS allows the estimation of the impact of ETS on the exposure compared to the model results with all indoor sources.

Visualisation tab

Visualisation is only available for grid results. The user can chose between mean/standard deviation and quantiles maps. Quantiles are currently fixed with the levels 0.01, 0.05, 0.10, 0.25, 0.50, 0.75, 0.90, 0.95, 0.99. It is possible to start Aguila automatically after LAMA finished if Aguila is installed on the PC where the program is executed. The maps are stored in the /output/vis folder under the folder name of the respective scenario.

If you cancel at the final configuration step or any other step, you have to start with the first step again and reenter all necessary information about the folder and/or database. If you keep the GUI at the final configuration panel opened, you can start several simulations for different countries etc. one after each other. The details and progress of the simulation are displayed in the command line window and are also written to the simulation log file (see next section).

# Final remarks & Known Issues

One major problem in LAMA is the Java OutOfMemoryError. This error occurs if too much data is in the Java program so that the main memory is not sufficient to store the Java objects. This can be partially solved by increasing the Java heap space, i.e. the amount of MB from the main memory that may be occupied by Java. This is already done when using the command to start the program:

java –Xmx1024M –jar C:\LAMA\LAMA.jar

The option –Xmx sets the available memory to 1024 Mb for Java. If you have enough main memory, you can increase this number.

Secondly, the time used by the calculations depends highly on the number of diaries and grid cells as input and the computing resources. For Germany, the country with the largest MTUS data set (201 cells, >60000 diaries), the calculation of 500 realisations for the diaries results took about 15 min. Calculating 1000 realisations at once for this country probably results in a OutOfMemoryError as there are too many dimensions for the array objects. Also the filling of the database can consume a considerable amount of time (~ 1.5 hours). It should be kept in mind that this program is not optimised for performance issues.

Thirdly, the resulting csv files for diaries can get very large, i.e. several hundreds of MB. This can cause problems when loading the data into other programs for further analysis. Therefore, it is recommended to use the database as storage for the results and use simple SQL requests to retrieve the data or only calculate subgroup averages.

# References

Airbase. (2008). "Airbase, European air quality database". Last accessed: November 2008.

Koutrakis, P.; Briggs, S. L. K.; Leaderer, B. P. (1992). Source Apportionment of Indoor Aerosols in Suffolk and Onondaga Counties, New York. Environ Sci Technol, 26:521-527.

Querol, X., Alastuey, A., Ruiz, C.R., Artiñano, B., Hansson, H.C., Harrison, R.M., Buringh, E. Et al (2004). “Speciation and origin of PM10 and PM2.5 in selected European cities”, Atmospheric Environment, 38:6547-6555

# Contact

Lydia Gerharz
Institute for Geoinformatics
University of Muenster
E-Mail: Lydia.Gerharz@uni-muenster.de

Aileen Yang

Norwegian Institute for Air Research,

Kjeller, Norway

E-Mail: Aileen.Yang@nilu.no

Miranda Loh

University of Arizona

Tucson, AZ

E-Mail: Miranda.loh@gmail.com

Alexandra Kuhn

Institute for Energy Economics and the Rational Use of Energy

University of Stuttgart

E-Mail: Alexandra.Kuhn@ier.uni-stuttgart.de

http://www.eea.europa.eu/data-and-maps/data/emep-grids-reprojected-by-eea

http://www.emep.int/

http://www.eea.europa.eu/data-and-maps/data/emep-grids-reprojected-by-eea